| Zagazig University Faculty of Engineering Computers and Systems Department | Course: CSE 323a - Measurements and Testing (1) 3rd Year Computers & Systems Engineering Final Exam – Thursday, January 15, 2015 |
| --- | --- |

| Total marks: 50 | Time allowed: 2 hrs | No of pages: 2 |
| --- | --- | --- |

Question 1: (16 Marks)

- A high-pass RC filter must drive 200 Hz noise down to 3%. Specify:

- the filter critical frequency,

- the value of the resistance, R, and

- the attenuation of a 20-kHz signal.

Hint: use a 0.01 µF capacitor.

- True or false:

- The higher the size of a resistor, the higher its power rating.

- The resolution of the digital multi-meters can be adjusted by adjusting its range.

- For pure sinusoidal waveform, the average value = 1.1 times its RMS value.

- Clamp meters can be used to measure small currents in the range of milli-amperes.

- In order to display a pulse with very short rise time, we must use an oscilloscope with high bandwidth.

- Explain the three-lead compensation in bridge circuits and its purpose.

Question 2: (16 Marks)

- What is the effect of loading in electrical circuits? Explain by an example, and give a solution.

- Explain briefly:

- a method for approximate measurement of capacitance.

- the measurement of phase using the X–Y mode on oscilloscope.

- frequency measurement using digital counter-timers.

Question 3: (18 Marks)

- Compare the following:

- Accuracy vs. precision.

- Static vs. dynamic characteristics of measuring systems.

- The following measurements were taken with an analog meter of the current ﬂowing in a circuit (the circuit was in steady state and therefore, although the measurements varied due to random errors, the current ﬂowing was actually constant):

{21.9 mA, 	22.1 mA, 	21.1 mA, 	21.7 mA, 	22.2 mA, 	21.8 mA}

Calculate:

- the mean value,

- the median value,

- the standard deviation,

- the standard error of the mean.

- use the standard error of the mean to express our confidence of the true mean.

- The resistance of a carbon resistor is measured by applying a d.c. voltage across it and measuring the current ﬂowing. If the voltage and current values are measured as 10 ± 0.1 V and 214 ± 5 mA respectively, express the value of the carbon resistor.

-------- All the best --------