| Zagazig University Faculty of Engineering Computers and Systems Department | Course: CSE 323a - Measurements and Testing (1) 3rd Year Computers & Systems Engineering Final Exam – Wednesday, January 27, 2016 |
| --- | --- |

| Total marks: 50 | Time allowed: 2 hrs | No of pages: 2 |
| --- | --- | --- |

Question 1: (13 Marks)

- What is the resonant frequency of a series RLC circuit if R = 30 kΩ, C = 2.2 nF, and L = 40 mH? What happens at resonance?

- A low-pass RC filter must drive 12 kHz noise down to 1%. Find:

- The filter critical frequency,

- The value of R (Use a 1 µF capacitor).

- The filter attenuation for a 60-Hz signal.

- If the ac bridge circuit shown is balanced, with R1 = 10 kΩ, R2 = 20 kΩ, R3 = 30 kΩ, and C1 = 10 μF, what are the values of R4 and C2?

Question 2: (14 Marks)

- Design a signal-conditioning to achieve the following relationship between the output and input voltages:

- Define:

- The resolution of a digital multi-meter,

- Pulse rise time,

- Oscilloscope trigger,

- Oscilloscope bandwidth.

- Explain briefly the operation of the electronic counter-timer used to measure the frequency of a sinusoidal signal. Illustrate your answer with the aid of block diagram.

Question 3: (13 Marks)

- Define:

  - The Insertion error.

  - The Hysteresis error.

  - Dynamic characteristics of measurement systems.

- The following five measurements are made of the output voltage from a high-gain ampliﬁer that is contaminated due to noise ﬂuctuations:

1.57, 1.54, 1.53, 1.51, 1.55

Determine the mean value and standard deviation. Hence estimate the accuracy to which the mean value is determined from these five measurements (i.e. give a confidence interval for the true mean).

Question 4: (10 Marks)

- The time constant of an RC circuit is given by RC seconds. If the value of the resistor is 10kΩ ±5% and the capacitor is 5μF±5%, estimate the minimum and maximum possible values of the time constant?

- Given the following data:

| x | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| y | 1.1 | 1.9 | 2.9 | 4.1 |

- Use linear regression to fit the data to a straight line.

- What is the sum of squared errors between the measured output y and the output predicted by the linear relationship you found in 1?

----- All best wishes -----

Answers

Question 1:

- f = 16,966 Hz.

- fc = 120 Hz, R = 1.326kΩ, attenuation = 89.44%.

- R4 = 60 kΩ, C2 = 5 μF.

Question 3:

- Mean = 1.54, standard deviation = 0.022, sem = 0.01. Hence, the true mean, with probability 95%, lies between 1.54±0.02.

Question 4:

- 45-55 mS.

- Y = x+1.

Sum of squared errors = 0.04.