| Zagazig University Faculty of Engineering Computers and Systems Department | Course: CSE 323a - Measurements and Testing (1) 3rd Year Computers and Systems Engineering Mid-term Exam – Wednesday December 3, 2014 |
| --- | --- |

| Total marks: 15 | Time allowed: 30 min | No of pages: 1 |
| --- | --- | --- |

Question 1: 	True or false:

| 1. | The maximum current in a series RLC circuit is obtained at the resonance frequency. | ( √ ) |  |
| --- | --- | --- | --- |
| 2. | Puff capacitors are polarized. | ( x ) |  |
| 3. | A system having a dominant time constant of 1 second will reach its new steady state, after a step input, in about 5 seconds. | ( √ ) |  |
| 4. | At the cutoff region, a second-order RC filter has a decay rate of 20 dB/decade. | ( x ) |  |
| 5. | At steady state, the output of a linear time-invariant system fed by a sinusoidal input will have the same amplitude as the input. | ( x ) |  |
| 6. | Null type bridges are used for calibration purposes. | ( √ ) |  |
| 7. | To display the full signal (dc and ac components) on an oscilloscope, the dc coupling is used. | ( √ ) |  |
| 8. | The AC voltage measurement, using a DMM, gives true rms value whatever the shape of the AC signal. | ( x ) |  |
| 9. | The oscilloscope is usable at frequencies up to its full bandwidth. | ( x ) |  |
| 10. | Trigger is used to stabilize the display of repetitive signals. | ( √ ) |  |

Question 2:

A temperature sensor outputs a voltage in the range of -50 to 250 mV as the temperature varies over its range.  Develop signal conditioning so that this range becomes 2 to 5 V.  The circuit must have very high input impedance.

Using the designed circuit, what is the output voltage obtained if the input is 50 mV?

Answer of Question 2

Let us develop an linear equation for the output in terms of the input

Vout = aVin+b

where a and b are to be found.

- For the two conditions we have in this problem we can write

2 = a(-0.050)+b

5 = a(0.250)+b

- Solving these two equations gives a = 10 and b = 2.5.

- Hence, the required equation is

Vout = 10Vin+2.5

- This can also be written as

Vout = 10(Vin+0.25)

- This looks like a differential amplifier with a gain of 10 and a fixed input of -0.25V to the inverting side. The following circuit shows how this could be done using an instrumentation amplifier.

For Vin  =50mV=0.05, the output will be Vout = 10(0.05+0.25) = 3V.