| Zagazig University Faculty of Engineering Computers and Systems Department | Course: CSE 323a - Measurements and Testing (1) 3rd Year Computers and Systems Engineering Mid-term Exam – November 21, 2015 |
| --- | --- |

| Total marks: 20 | Time allowed: 45 min | No of pages: 1 |
| --- | --- | --- |

Question 1 (8 Marks):

True or false:

- Capacitors larger than 1 μF are polarized.

- The larger is the time constant, the faster is the response.

- The gain in dB, for an amplifier, is negative while it is positive for an attenuator.

- The frequency range from 1 to 11Hz is called a frequency decade.

- At steady state, the output of a linear time-invariant system fed by a sinusoidal input will have the same amplitude as the input.

- In bridge circuits, the relationship between the change in resistance and the corresponding change in the output voltage is nonlinear.

- A dummy gauge is used to compensate for temperature effects.

- The loading effect is negligible with DMMs and oscilloscopes.

Question 2 (12 Marks):

- Compare null and deflection-type bridges in terms of their operation and usage.

- Draw a voltage-to-current converter to convert 1-5V range into 4-20mA range.

- Explain briefly the operation of the counter-timer used to measure phase difference between two sinusoids. Illustrate your answer with the aid of block diagram.