| Zagazig University Faculty of Engineering Computers & Systems Department | Elective (4): CSE 417 – Computer-based Control Systems 4th Year Electronics & Communications Engineering Final Exam – Wednesday, January 15, 2014 |
| --- | --- |

| Total marks: 50 | Time allowed: 2 Hrs | No of pages: 2 |
| --- | --- | --- |

Question 1: (20 Marks)

The open-loop unit step response of a thermal system is shown.

- Obtain the transfer function of this system.

- Use the Ziegler–Nichols tuning rules to design a proportional plus integral (PI) controller. Hint: ZN settings for PI controller are:

Kp=0.9T1KTD,  Ti=3.3TD

where T1 is the time constant, TD is the process time delay, and K is the process gain.

- Draw the block diagram of the closed loop system (in the s-domain).

- Transform the plant in part (a), with a zero-order hold preceding it, into discrete domain. Use a sampling period T = 1 sec.

Question 2: (20 Marks)

The open-loop transfer function of a system together with a zero-order hold is given by:

HGz=1z-0.4.

It is required that the closed-loop system has only a single pole at z = 0.2. The steady-state error to a step input should be 0.1. Assume that T = 1 sec.

- Deduce the desired closed-loop transfer function.

- Design the digital controller to achieve the desired closed-loop transfer function.

- Find the step response of the closed-loop system using partial fraction expansion.

Question 3: (10 Marks)

Consider the following model:

y(k) = a y(k −1) + b u(k −1).

An experiment is done on the system to estimate the parameters, a and b. The following input-output data were recorded:

| t | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| u(t) | 0 | 1 | 0 | 1 | 0 |
| y(t) | 0 | 0 | 2 | 1 | 3 |

- Determine the least squares estimate of a and b.

- Is the identified system stable?

------- with all best wishes ------