| Zagazig University Faculty of Engineering Computers & Systems Department | Elective (4): CSE 417 – Computer-based Control Systems 4th Year Electronics & Communications Engineering Final Exam – Wednesday, January 14, 2015 |
| --- | --- |

| Total marks: 50 | Time allowed: 2 hrs | No of pages: 2 |
| --- | --- | --- |

Question 1: (20 Marks)

Consider the following discrete-time closed-loop system, with a sampling period, T = 1 second:

- Find the discrete-time transfer function, G(z), of the plant, G(s).

- Find the discrete-time closed-loop transfer function.

- What is the maximum value of K that keeps the closed-loop system stable?

- Using partial fraction, obtain the output sequence c(nT) for a unit-step input, assuming that K=1.5.

- Find the steady-state error for a unit-step input, assuming again that K=1.5.

- Design a deadbeat controller for the process G(z), where K=1.5. What is the type of the controller obtained?

Question 2: (8 Marks)

Design a digital controller, D(z), such that the poles of the following closed-loop system are placed at z1,2 = 0.4 ± j0.4 in the z-plane. The steady-state error in the step response must be zero.

Question 3: (11 Marks)

- True or false:

  - Increasing the proportional controller gain speed up the system but reduces its stability margin. (T)

  - The derivative controller action depends on the past values of the error. (F)

  - The sampling period must be small enough to keep the system stable. (T)

  - The stability region in the z-plane is the left-half side. (F)

  - Higher-order plants can be well-approximated by first-order system with time delay. (T)

- The digital control algorithm should be synchronized so that it runs exactly at the sampling instants. Explain, briefly, two methods for this purpose.

Question 4: (11 Marks)

Consider the following model:

An experiment is done on the system to estimate the parameters, a and b. The following input-output data were recorded:

| t | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| u(t) | 1 | 2 | 1 | 1 | 0 |
| y(t) | 0 | 0 | 2 | 1 | 3 |

- Using the given data, write down the equations in matrix form.

- Determine the least squares estimate of a and b.

- Find the model output and the residuals.

- What is the transfer function of model obtained?

-------- All the best --------

Exam Answer

Q1

(a)

(b)

- The characteristic equation is

Hence the pole is

For stability,

- The output is

- The steady state error:

This can be calculated using two methods:

First noting that c(∞)=1, and that e(∞) = r(∞) – c(∞) = 1-1=0.

The second method:

(f)

For the controller to be realizable,

Which is a proportional controller.

Q2

As the process has delay of two samples,

For a step input

Q4

(a)

(b)

- The model output

The residual is

- The transfer function

After using the values of the estimates, the difference equation