| Zagazig University Faculty of Engineering Computer & Systems Department | CSE 421 – Digital Control 4th Year Computer & Systems Eng. Final Exam – Sunday, January 24, 2016 |
| --- | --- |

| Total marks: 100 | Time allowed: 3 hrs | No of pages: 2 |
| --- | --- | --- |

Question 1: (20 Marks)

- Explain, briefly, one method to run the digital control algorithm exactly at the sampling instants.

- Consider a signal of frequency 10Hz, sampled at a rate of fs = 50Hz. Assume that there is a noise component at 35 Hz. What are the frequencies that appear at the sampled signal? Does the aliasing problem occur? Why? How to solve it?

- Explain what derivative kick is when a PID controller is used. How can derivative kick be avoided?

Question 2: (20 Marks)

Given the following discrete-time control system (with sampling period T=1sec):

- Find the transfer function Y(z)/R(z).

- What is the maximum value of K that keeps the closed-loop system stable?

Question 3: (20 Marks)

- Design a deadbeat controller D(z) for the following closed-loop system:

- Repeat part (a) but now design a Dahlin controller D(z) where the desired closed loop time constant is 1 second.

Question 4: (15 Marks)

Consider the following model:

An experiment is done on the system to estimate the parameters, a and b. The following input-output data were recorded:

| t | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| u(t) | 1 | 1 | 0 | 0 |
| y(t) | 0 | 2 | 1 | 0 |

- Using the given data, write down the equations in matrix form.

- Determine the least squares estimate of a and b.

- Find the model output and the residuals.

- What is the transfer function of the model obtained?

- Is the model stable? Why?

Question 5: (25 Marks)

- Find a discrete-time state space description of the following system (using a sampling period of 1 sec).

- Given the following discrete-time system:

- Check if the system is observable and controllable.

- Design a state feedback controller with closed-loop eigenvalues of 0.5 and -0.5.

- Design a state observer with eigenvalues of 0.2 and -0.2.

-------- All the best --------

Exam Answer

Q2

(a)

For stability, we have one closed loop poles:

z = 0.368-0.632K

|0.368-0.632K|<1

-1 < 0.368-0.632K < 1

-1.368 < -0.632 K < 0.632

  - > K > -1

Question 3

(a)

(b)

The dahlin controller:

The desired closed-loop transfer function

Q4

(a)

(b)

- The model output

The residual is

- The transfer function

After using the values of the estimates, the difference equation

The system is stable because the pole is inside the unit circle.